Wiring like this :

Upload Marlin provided in « Thing Files »

In configuration.h, you can make settings :

#define LINKAGE1 120//155 //mm  // Length of outer support arm     Measure arm lengths precisely and enter

#define LINKAGE2 120 //180 //mm

#define CENCER_OFFSET 0 //65 //mm

#define HEAD_OFFSET 60 //50//mm

// SCARA tower offset (position of Tower relative to bed zero position)

// This needs to be reasonably accurate as it defines the printbed position in the SCARA space.

#define SCARA_OFFSET_X 160 //130 mm

#define SCARA_OFFSET_Y 0 //mm

#define SCARA_OFFSET_Z -35 //-40 //65 //mm

Manually Home Position : the boom is 90 degrees to the horizontal, the arm is parallel to the horizontal line

Connect with Pronterface

Enter G93 X90 Y0 Z0

Axis :

Enter m114 : Pronterface tell you Hot End position : X=20 , Y=0, Z=155 (with my settings)

Clic on X+10 icon : Hot End must go straight in X positive

Enter m114 : you should have Hot End position : X=30 , Y=0, Z=155 (with my settings)

Enter g1 z0 : the Hot End go to z=0

Enter g1 x0 : the Hot End go to x=0

Load .gcode (see in "Thing Files"),

Print...